Waltham Parish Council

EST. 1894

https://waltham-pc.gov.uk                                                    https://clerk@waltham-pc.gov.uk

Report to Council on Highways Improvement Plan Priorities

Figure 1 Topography of Waltham Parish

Source: Parish Online XMap software

0    Residential dwellings                                                           0 Commercial premises

Introduction.

Consultation with parishioners through the Parish Plan questionnaire revealed that Sustainable Transport was regarded as a high priority. The intention of this report is to highlight the main areas of concern, suggested action, and approximate costs in order to inform Member decisions on Highways Improvement Plan priorities and also discussions on budget setting for 2024/25 and beyond.

Priority 1. Speed Reduction Options

- Traffic Speed Survey. This was conducted in collaboration with KCC in February/March 2021 on Kake Street, the bus route through Waltham Village. The report suggested that while some instances of exceeding speed limits had occurred, they were not regarded as a significant problem. Continuation of parishioner concerns suggests the need for a repeat survey.

The cost for a single ATC tube survey is around £85 for a week of data collection, which allows reports such as traffic volume, speed, and Vehicle classification.

- Gate Sign. These need to be installed where there is an existing speed limit change or at a suitable point at the entry to a village to achieve the maximum effect.

In 2022 WPC successfully applied for a Capital Grant from CCC for 2023/24, with WPC contributing 50% towards the estimated cost of £1,525. In the intervening period the basic cost has risen to £1,700. It is recommended that the additional cost is met from Capital reserves.

- Speed Limit Change. At a time when many authorities are reducing speed limits down to 20mph in built up urban and rural areas, the implementation of a 20mph  restriction in the village and hazardous access lanes requires consideration. Guidance is offered by the Department of Transport (2013) but the increase in vehicular ownership in the past 10 years justifies a review of local circumstances. It is suggested that a 20mph zone is appropriate as an alternative to the existing 30mph limit.

The typical basic cost of £6,100 may be prohibitive, with a Traffic Regulation Order from £2,850, and  5 x £650 entry signs & posts. A significant build-up of restricted Capital Reserves would be required over two to three years depending on the level of grant funding achieved and/or an increase in the Precept demand.

- 30mph Toolkit.  The existing speed restrictions may be underpinned  with education, training, publicity, and enforcement. KCC offers free advisory information; road user tips; social media messages ; posters; car and bin stickers and road banners.

Priority 2.  Road Hazard Reduction.

Waltham parish has grass verges on most of its roads, and the lack of pavements presents a particular problem for pedestrians. There are three commercial stables in the parish and horse riding is a popular pastime. In the Parish Plan questionnaire, the need for a reduction in vehicle hazards by increasing driver awareness was regarded as  essential.

While the risk of “sign blindness” is recognised, together with visual intrusion in an AONB, the need to address safety issues and the benefits of providing highway users with appropriate information should be taken into consideration.

The cost of a basic warning sign starts from about £260, a combination of children and equine signs significantly more, so for signage at the 6 access points to the parish the capital costs would be in the region of £2,000 to £3,000.

Priority 3. Road Treatment during Adverse Weather

Because of the local topography (figure 1) and the unclassified lanes serving much of the parish, Waltham is particularly affected during snow or icy conditions.

KCC assumes responsibility in its Winter Service Policy (2020) for precautionary treatment of the 0.5-mile bus route through the village along Waltham Road; Kake Street, and Woods Hill along the top of the plateau, but the narrow lanes serving Anvil Green; Richdore Road; down to Duckpit and up to Stone street remain untreated apart from the provision of salt bins.

To ameliorate the effect of adverse weather to those households and business premises in isolated areas, WPC has a local Adverse Weather Policy which is reviewed annually and is concerned with snow clearing in emergencies. As with KCC there is no specific budget for this because of the variable nature of the weather. However, an indicative sum of £800 is identified in  unrestricted reserves, and an informal agreement has existed with a local farmer to undertake the work. This arrangement does not meet the councils risk assessment criteria and more formal arrangements are needed for the coming winter season.

Qualitative feedback from parishioners suggests that those arrangements should include precautionary salting during icy conditions, but this raises concerns about the potential cost, and effects of chemical pollution, creating a tension between road safety and environmental concerns, as indicated by Raworth, K. (2017) [p.44] in her work on human wellbeing and the ecological ceiling.

It is suggested that the potential for a joint arrangement with KCC should be explored through discussions on the HIP, using WPC’s General Power of Competence to make a financial contribution to the  County’s Winter Service programme for the parish, the level of service to be determined by a maximum budget of £3,000 to be held in general contingency funds.

A  Delivery Plan.

Delivery Timetable

| Priority | 2023/24 | 2024/25 |  | 2025/26 | 2026/27 | 2027/28 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| - Speed Reduction |  |  |  |  |  |  |  |
| * Gate sign (Capital reserve) | £1,700 |  |  |  |  |  |  |
| * Traffic survey (Revenue) | £85 |  |  |  |  |  |  |
| * 30mph Toolkit | £0 |  |  |  |  |  |  |
| * Speed Limit Change (Capital reserve) |  | £2,000 |  | £2,000 | £ 2,000 | Implement |  |
| - Hazard reduction |  |  |  |  |  |  |  |
| * Equine/Children signs (Capital Reserve) |  | £3,000 |  | £0 | £0 | £0 |  |
| - Adverse Weather Road Treatment |  |  |  |  |  |  |  |
| * Adverse Weather Policy Agreement (Revenue) |  | £3,000 |  | £3,000 | £3,000 | £3,000 |  |
| Financial Implications |  |  |  |  |  |  |  |
| Estimated Cost |  |  |  |  |  |  |  |
| Capital (restricted reserves) | £1,700 | £5,000 |  | £2,000 | £2,000 |  |  |
| Revenue (unrestricted Reserves) | £85 | £3,000 |  | £3,000 | £3,000 | £3,000 |  |
| Total | £1,785 | £8,000 |  | £5,000 | £5,000 | £3,000 |  |
| Budget Forecast |  |  |  |  |  |  |  |
| Actual / Estimated Annual Budget | £7,161 | £9,661 |  | £8,000 | £8,000 | £8,000 |  |

NB:

- It is expected that the carry forward balance from 2023/4 accounts will be circa £6,000 and will cushion the 2024/25 anticipated spend.

- A 2024/25 Capital Grant application of £1,500 has been submitted to Canterbury City Council to support Priority 2.

WPC/asm/22:09:2023